## Add-on Feature: First-Run “Commissioning Password” (One-time only)

### 1) Behavior (User Experience)

First time boot/run (नई machine):

- Software auto-open

- Screen: “First Time Setup (Commissioning)”

- User को Commissioning Password डालना होगा

- सही password के बाद:

  - Setup wizard खुलेगा (basic settings: mode, control source, PLC enable, safety limits, etc.)

  - अंत में “Complete Setup”

- इसके बाद software normal mode में आ जाएगा और फिर startup पर password नहीं पूछेगा

Next boots (power fail, restart):

- auto-open → self-check → normal operator screen

- commissioning password नहीं पूछेगा

### 2) Engineer Mode Password अलग रहेगा (Recommended)

One-time commissioning password केवल “new machine enable” के लिए होगा।

लेकिन Engineer Screen (settings/calibration) के लिए:

- password हर बार (या session-based) रहना चाहिए

- ताकि operator गलती से calibration/settings न बदल दे

So, दो अलग security layers:

- Commissioning Lock (one-time) – only on fresh install/new device

- Engineer Password (always) – for advanced settings

### 3) How software decides “first run है या नहीं”

Software एक flag store करेगा:

- commissioned = false (default)

- जब commissioning complete होता है → commissioned = true

Storage options (recommended):

- SQLite में device_state table

- या config/device_state.json जैसी file (but SQLite more robust)

### 4) Password handling (Security rules)

- Password plain text में store नहीं करेंगे

- Store करेंगे: salted hash (bcrypt/argon2)

- First-run पर default password allow हो सकता है, लेकिन best practice:

  - first-run में “Set new commissioning password” prompt

  - फिर old default disable

आपकी requirement के हिसाब से simplest option:

- Manufacturer/Owner fixed commissioning password (आप define करोगे)

- First-run पर verify → commissioned true → done

### 5) Factory Reset / Re-Commission (Important)

कभी-कभी machine shift होती है या system rebuild होता है। इसलिए:

- Engineer screen में (password protected): “Re-Commission Device” button

- या physical method:

  - SD card image reset

  - “RESET” file USB से डालकर commissioning reset

  - GPIO jumper pin short करके reset (industrial-friendly)

Reset के बाद:

- commissioned = false

- next boot पर फिर commissioning password माँगेगा

## Plan में इसे कहाँ add करेंगे (Updated sections)

### A) Startup Flow (Update)

- BOOT → SELF_CHECK →

  - अगर commissioned=false → COMMISSIONING_WIZARD (password required)

  - else → SAFE_IDLE / AUTO_RUN (mode rules)

### B) UI Screens (Update)

- Commissioning Screen

  - Password field

  - “Unlock” button

- Commissioning Wizard

  - Mode select (Tabletop/Inline)

  - Control source (Local/Remote/Mixed)

  - Remote enable

  - PLC settings (Modbus on/off, IP/port view)

  - Safety limits (max current, air-cal current)

  - “Complete Setup”

### C) PLC behavior during commissioning

- Commissioning state में:

  - Drive default OFF

  - PLC outputs status: READY=0, SELF_CHECK_FAIL as per check, and add one bit:

    - STATUS_WORD bit12 = COMMISSIONING_REQUIRED

  - PLC commands ignore (या limited) until commissioned true

## Minimal Spec (One-line)

“New machine पर पहली बार run में commissioning password लगेगा; once setup complete हो जाए तो फिर कभी startup पर password नहीं माँगेगा, लेकिन engineer settings हमेशा password protected रहेंगी।”